# Gobind Sugar Mill, Aira

Weekly Leadership Review

Week: 28th Dec. to 03th Jan. 2026 | Compared with: 21st Dec to 27th Dec 2025

Season Day: 70

## PART 1 – INSIGHT NARRATIVE

### 1. Executive Summary

CRUSHING PERFORMANCE: Total sugarcane crush decreased to 6.91 LQ from 7.51 LQ last week. Highest crush with one edge of shredder tips reached 32.45 L Qtls (previous week 26.23 L Qtls). Capacity utilization declined to 98.79% from 107.29%. Average monthly crush rate was 104745 Qtls per day.

RECOVERY: Sugar recovery marginally improved to 10.44% from 10.43%. Till date recovery maintained at 9.89%. Pol in cane increased slightly to 13.02% from 13.01%.

LOSSES: Total losses increased to 2.58% from 2.52% mainly due to Pol in final molasses rising to 1.97% from 1.91%. Losses in bagasse and filter cake stayed flat.

POWER: Power generation slightly declined to 63.66 LU from 64.71 LU. Power export marginally down at 36.80 LU from 36.92 LU. Overall power plant availability remains above 97%, with some turbines slightly below 100%.

DISTILLERY: Ethanol production steady at 8.32 LL vs 8.42 LL. Capacity utilization remained high at 95.11% from 96.20%. Distillery experienced a planned shutdown on 03.01.2026 with steam supplied from 70 TPH boiler.

SUGAR QUALITY: DS House ICUMSA improved: M-sugar at 85 vs 92; S-sugar at 88 vs 98. RS House quality declined slightly: L-sugar at 32 vs 29; M-sugar at 32 vs 29.

CAPEX / PROJECTS: Mud Removal System nearing completion with all mechanical, electrical, and instrumentation work completed. GEA & Chemical system expected at site by 05.01.2026 for commissioning.

EHS & SAFETY: No accidents reported this week. Road safety and BBS trainings conducted. Fire mock drill completed in Distillery. Implementation of reflection/radium tapes and random ethanol truck checks ongoing.

### Risks & Watch-outs for the Coming Week

• Decrease in gate and center indent maturity poses supply risk.

• Increasing downtime in engineering requires attention to prevent crushing losses.

• Pol in final molasses increasing; monitor losses closely.

• Planned distillery shutdown requires careful steam management.

• Ongoing commissioning risk for Mud Removal System.

• Continued KCC loan and no ratoon issues affecting indent fulfillment.

• Repeat truck involved in accidents; must improve road safety training.

### TABLE A – Quantitative Discussion

| KPI | Current Week | Last Week | Till Date | WoW Change |
| --- | --- | --- | --- | --- |
| Cane Crushed (LQ) | 6.91 | 7.51 | 63.14 | -0.60 LQ |
| Sugar Recovery (%) | 10.44% | 10.43% | 9.89% | +0.01 pp |
| Sugar Produced (LQ) | 0.61 | 0.65 | 5.75 | -0.04 LQ |
| Capacity Utilization (%) | 98.79% | 107.29% | 100.08% | -8.50 pp |
| Total Losses (%) | 2.58% | 2.52% | 2.11% | +0.06 pp |
| Pol % Bagasse | 1.71% | 1.71% | 1.75% | Flat |
| DS ICUMSA (M-sugar) | 85 | 92 | 105 | -7 |
| Power Generation (LU) | 63.66 | 64.71 | 636.12 | -1.02 LU |
| Power Export (LU) | 36.80 | 36.92 | 375.73 | -0.12 LU |
| Power PLF - 30.85 MW (%) | 96.07% | 96.69% | 97.05% | -0.62 pp |
| 150 TPH Boiler (TPH) | 148.2 | 151.2 | 150.4 | -3.0 TPH |
| Ethanol Produced (LL) | 8.32 | 8.42 | 77.53 | -0.10 LL |
| Distillery PUF (%) | 95.11% | 96.20% | 95.33% | -1.09 pp |
| Fermentation Eff. (%) | 91.40% | 91.17% | 90.95% | +0.23 pp |
| Steam Consumed (Kg/BL) | 4.06 | 4.02 | 4.10 | +0.04 |
| Power Consumed (KWH/BL) | 0.429 | 0.434 | 0.432 | -0.005 |

### TABLE B – Qualitative Discussion

| Area | Current Week Narrative | Change vs Last Week |
| --- | --- | --- |
| Engineering | Hours utilized decreased to 92.93%. Downtime increased to 3.53 hrs from 0.32 hrs due to overload related stoppages and scheduled general cleaning. Shredder tips replaced with new, side hammers partially replaced. Total crushing 32.45 L Qtls with one edge of shredder tips. | Operated first time above 25 L Qtls with one edge of shredder tips previous week (26.23 L Qtls). Downtime increased and general cleaning scheduled. |
| Cane Planning | Daily indent rate increased to 1.11 LQ. Gate and center indent maturity declined to 81% and 71% respectively. Affected cane reduced to 0.5%. Ongoing issues with KCC loans, dead growers, and uprooted ratoons continue as top reasons for indent failure. | Gate indent maturity declined from 95% to 81%; center indent maturity declined from 100% to 71%. Affected cane reduced from 1.1% to 0.5%. |
| Production | Sugar production decreased slightly to 0.61 LQ. DS House ICUMSA M-sugar improved to 85 from 92. Some quality parameters stable. Steam consumption increased to 459 Kg/Qtl sugar. Recovery stable at 11.18%. | Sugar production decreased by 0.04 LQ. DS House ICUMSA M-sugar improved; steam consumption increased. |
| Power | Power generation decreased to 63.66 LU. Power export nearly flat at 36.80 LU. Turbine PLF marginally decreased. Boiler capacities stable but 35 TPH boiler nearly at threshold capacity. | Power generation declined by 1.02 LU. Power export nearly flat. |
| Distillery | Ethanol production slightly declined to 8.32 LL. Distillery had a planned shutdown on 03.01.26 with alternate steam supply. Efficiency metrics stable. | Capacity utilization declined from 96.2% to 95.11%. Planned shutdown during the week. |
| Quality/EHS | Total losses increased slightly to 2.58% with Pol in molasses rising. Fire and safety trainings ongoing. No accidents reported this week, 23 incidents cumulative in 9 weeks. 4 BBS sessions conducted. | Total losses up by 0.06 pp. Safety trainings and no lethality accidents reported. |
| Capex | Mud Removal System nearing completion with piping, electrical and instrumentation completed. Commissioning expected 05.01.26. | Progress from 90% to near completion; commissioning imminent. |

This review is based exclusively on numbers and narratives disclosed in the weekly PDF slides.